EXP. NO: 16 SIMULATION AND ANALYSIS OF UNIFORM AND NON-UNIFORM

QUANTIZATION USING MATLAB

NAME:V.RUSHMITHA

DATE:09/10/2026

Objectives

i) To implement and analyze uniform and non-uniform quantization techniques for an analog

signal using MATLAB by converting the continuous-amplitude signal into discrete

quantization levels.

ii) To compare the performance of uniform and non-uniform quantization by analyzing their

quantized waveforms and quantization errors, and evaluate their effectiveness in representing

signals with different amplitude variations.

Procedure

1. Open MATLAB and create a new script file.

2. Define the simulation parameters, including the sampling frequency, time vector, signal

amplitude, signal frequency, and the number of quantization levels.

3. Generate the analog input signal using a sinusoidal waveform.

4. Implement uniform quantization by dividing the signal amplitude range into equal quantization

intervals and obtain the uniformly quantized signal.

5. Calculate the quantization error by finding the difference between the original analog signal and

the uniformly quantized signal.

6. Implement non-uniform quantization using a companding technique (such as μ-law or A-law)

followed by quantization.

7. Recover the non-uniformly quantized signal by applying the corresponding expansion process.

8. Calculate the quantization error for the non-uniform quantized signal.

9. Plot the original analog signal, uniformly quantized signal, non-uniformly quantized signal,

uniform quantization error, and non-uniform quantization error using MATLAB subplots.

10. Compare the performance of uniform and non-uniform quantization by analyzing the quantized

waveforms and the corresponding quantization errors.

11. Evaluate the effectiveness of both quantization techniques in representing signals with different

amplitude variations.

12. Interpret the simulation results and conclude the advantages of non-uniform quantization over

uniform quantization for low-amplitude signals.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

% Input Signal

t = 0:0.001:1;

x = sin(2*pi*5*t);

% Uniform Quantization (8 Levels)

L = 8;

x_uni = round((x+1)*(L-1)/2); x_uni = (x_uni*2/(L-1))-1;

% Non-Uniform Quantization (μ-law)

mu = 255;

x_comp = sign(x).*log(1+mu*abs(x))/log(1+mu);

x_non = round((x_comp+1)*(L-1)/2);

x_non = (x_non*2/(L-1))-1;

% Plot Results

figure;

subplot(3,1,1)

plot(t,x)

title('Original Signal')

grid on

subplot(3,1,2)

stairs(t,x_uni)

title('Uniform Quantization')

grid on

subplot(3,1,3)

stairs(t,x_non)

title('Non-Uniform Quantization (\mu-law)')

grid on

OUTPUT GRAPH

Result:

i) The uniform and non-uniform quantization techniques were successfully implemented in

MATLAB, and the corresponding quantized signals and quantization error waveforms were

obtained.

ii) The performance comparison showed that non-uniform quantization produced lower

quantization error for low-amplitude signals than uniform quantization, resulting in improved

signal representation and better overall quantization efficiency.