EXP. NO: 17 PERFORMANCE EVALUATION OF DIGITAL COMMUNICATION

SYSTEMS USING EYE DIAGRAM ANALYSIS UNDER ADDITIVE WHITE GAUSSIAN NOISE (AWGN) USING MATLAB

NAME:V.RUSHMITHA

DATE: 09/10/2026

Objectives

1. To generate a digital communication signal in MATLAB and transmit it through an

Additive White Gaussian Noise (AWGN) channel.

2. To obtain and analyze the eye diagram of the received digital signal for different noise

conditions.

3. To measure eye-opening parameters such as eye height, eye width, crossing level and

timing variation.

4. To study the effect of decreasing SNR / Eb/N0 on inter-symbol interference, noise margin

and timing performance.

5. To compare the transmitted and received signal characteristics and evaluate

communication-system performance.

Procedure

1. Open MATLAB and create a new script file.

2. Set the number of bits/symbols, samples per symbol, modulation scheme, symbol rate and sampling

parameters.

3. Generate a random binary sequence and map it to a digital waveform. For a basic experiment, use

BPSK with symbols −1 and +1.

4. Generate the transmitted waveform using an appropriate pulse-shaping method or rectangular NRZ

pulses.

5. Pass the waveform through an AWGN channel. Repeat the experiment for several SNR/Eb/N0

values, such as 20 dB, 10 dB, 5 dB and 0 dB.

6. Plot a portion of the transmitted and received waveforms in the time domain.

7. Construct the eye diagram by superimposing overlapping signal segments over two symbol periods.

8. Measure or estimate eye height, eye width, crossing level and timing uncertainty from the eye

diagram.

9. Compare the eye diagrams for different noise levels and observe the reduction in vertical and

horizontal eye opening.

10. Relate the observed eye-pattern changes to noise margin, sampling-time tolerance and probability

of symbol decision errors.

11. Record the results and calculate percentage deviation/error where a theoretical reference is

available.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

%% Parameters

N = 100; % Number of bits sps = 20; % Samples per symbol

SNR = 10; % Signal-to-Noise Ratio (dB)

%% Generate Random Binary Data

data = randi([0 1],1,N);

%% NRZ Encoding (0 -> 0, 1 -> 1)

tx_signal = repelem(data,sps);

%% Pass Signal through AWGN Channel

rx_signal = awgn(tx_signal,SNR,'measured');

%% Time Axis

t = (0:length(tx_signal)-1)/sps;

%% Plot Results

figure;

% Original Binary Data

subplot(4,1,1)

stairs(data,'LineWidth',1.5)

grid on

title('Original Binary Data')

xlabel('Bit Number')

ylabel('Bit')

ylim([-0.2 1.2])

% Transmitted NRZ Signal

subplot(4,1,2)

plot(t,tx_signal,'LineWidth',1.5)

grid on

title('Transmitted NRZ Signal')

xlabel('Time')

ylabel('Amplitude')

ylim([-0.5 1.5])

% Received Signal after AWGN

subplot(4,1,3)

plot(t,rx_signal,'r')

grid on

title(['Received Signal with AWGN (SNR = ',num2str(SNR),' dB)'])

xlabel('Time')

ylabel('Amplitude')

% Eye Diagram

subplot(4,1,4)

eyediagram(rx_signal,2*sps)

title('Eye Diagram of Received Signal')

OUTPUT GRAPH

Result:

i) Successfully generated an NRZ digital communication signal and transmitted it through an

AWGN channel.

ii) Observed that the received signal was affected by Gaussian noise, resulting in signal

distortion and bit errors.

iii) Successfully plotted the eye diagram of the received signal using MATLAB.

iv) Observed that the eye opening decreased due to AWGN, indicating reduced signal quality

and increased probability of bit errors.