EXP. NO: 6 : GENERATION AND ANALYSIS OF FREQUENCY MODULATED (FM) AND OF PHASE MODULATED (PM) SIGNALS USING MATLAB

NAME:V.RUSHMITHA

DATE:

Objectives:

To generate and analyze Frequency Modulated (FM) and Phase Modulated (PM) signals using

MATLAB by varying the instantaneous frequency and phase of a carrier signal in accordance

with the message signal.

Procedure

1. Open MATLAB and create a new script file.

2. Define the simulation parameters, including the sampling frequency, time vector, message

frequency, carrier frequency, message amplitude, and carrier amplitude.

3. Generate the message signal using a low-frequency sinusoidal waveform.

4. Generate the carrier signal using a high-frequency sinusoidal waveform.

5. Generate the Frequency Modulated (FM) signal by varying the instantaneous frequency of the

carrier in accordance with the message signal and the selected frequency modulation index.

6. Generate the Phase Modulated (PM) signal by varying the instantaneous phase of the carrier

according to the message signal and the selected phase modulation index.

7. Plot the message signal, carrier signal, FM signal, and PM signal using MATLAB subplots.

8. Observe and compare the time-domain waveforms of the FM and PM signals with the original

message signal.

9. Analyze the variation in the instantaneous frequency of the FM signal and the instantaneous

phase of the PM signal due to changes in the message signal.

10. Compare the characteristics of FM and PM signals by studying their modulation behavior and

waveform variations.

11. Verify the successful generation of FM and PM signals based on the simulation results.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

Am=1;

Ac=1;

fm=5;

fc=50;

beta=5;

kp=5;

fs=1000;

t=0:1/fs:1;

m = Am*cos(2*pi*fm*t); % Message signal c = Ac*cos(2*pi*fc*t); % Carrier signal

fm_sig = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t)); % FM

pm_sig = Ac*cos(2*pi*fc*t + kp*m); % PM

Df = beta*fm; % Frequency deviation

fprintf('Frequency Deviation = %d Hz\n',Df);

fprintf('FM Modulation Index = %d\n',beta);

subplot(4,1,1), plot(t,m), title('Message Signal')

subplot(4,1,2), plot(t,c), title('Carrier Signal')

subplot(4,1,3), plot(t,fm_sig), title('FM Signal')

subplot(4,1,4), plot(t,pm_sig), title('PM Signal')

OUTPUT GRAPH

Result

The Frequency Modulated (FM) and Phase Modulated (PM) signals were successfully

generated using MATLAB by varying the instantaneous frequency and phase of the carrier

signal according to the message signal.

• Frequency Deviation (Δf) = 25 Hz

• FM Modulation Index (β) = 5