EXP NO: 2 CHARACTERISTICS STUDY OF SIDE BAND POWER AND

FREQUENCIES IN AM USING MATLAB

NAME: V.RUSHMITHA

DATE:07/10/2026

Objectives:

i) To generate an Amplitude Modulated (AM) signal in MATLAB using specified carrier and

modulating signal parameters.

ii) To determine and verify the upper sideband (USB) and lower sideband (LSB) frequencies of

the AM signal using frequency spectrum analysis in MATLAB.

iii) To calculate the carrier power, sideband power, and total transmitted power of the AM signal

for a given modulation index using MATLAB.

iv) To analyze the effect of varying the modulation index on sideband power distribution and

overall transmission efficiency through MATLAB simulation.

Objective 1:

To generate an Amplitude Modulated (AM) signal in MATLAB using specified carrier and

modulating signal parameters

Procedure

1. Open MATLAB and create a new script for AM signal generation and analysis.

2. Specify the sampling frequency, carrier frequency, modulating frequency, carrier amplitude and

modulation index.

3. Generate the modulating signal using, m(t)=Amcos(2πfmt)

4. Generate the carrier signal using, c(t)=Accos(2πfct)

5. Generate the AM signal using, s(t)=Ac[1+mcos(2πfmt)]cos(2πfct).

6. Plot the modulating signal, carrier signal and AM signal in the time domain.

7. Calculate the theoretical frequencies of the lower and upper sidebands.

8. Obtain the frequency spectrum of the AM signal using MATLAB FFT analysis.

9. Identify the carrier, LSB and USB peaks from the frequency spectrum.

10. Verify the theoretically calculated frequencies with the frequencies observed from the MATLAB

spectrum.

11. Calculate the carrier power using the specified load resistance.

12. Calculate the power contained in each sideband and the total transmitted power.

13. Calculate the transmission efficiency for the selected modulation index.

14. Repeat the simulation for different modulation-index values such as m=0.2, 0.5, 0.8 and 1.0.

15. Observe the change in sideband power and transmission efficiency as the modulation index is

varied.

16. Record the results and compare the theoretical and MATLAB-simulated values.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

Am=1;

fm=500;

Ac=5;

fc=5000;

fs=20*fc;

t=0:1/fs:4/fm;

m = Am*sin(2*pi*fm*t);

c = Ac*sin(2*pi*fc*t);

mu = Am/Ac;

am = Ac*(1+mu*sin(2*pi*fm*t)).*sin(2*pi*fc*t);

dem = abs(hilbert(am)); dem = dem-mean(dem);

N=length(am); f=linspace(-fs/2,fs/2,N);

AM_FFT=abs(fftshift(fft(am)))/N;

USB=fc+fm; LSB=fc-fm;

Pc=Ac^2/2; Psb=(mu^2*Pc)/4;

figure('Name','AM Modulation','NumberTitle','off');

subplot(3,2,1), plot(t,m), title('Message'), grid on

subplot(3,2,2), plot(t,c), title('Carrier'), grid on

subplot(3,2,3), plot(t,am), title('AM Signal'), grid on

subplot(3,2,4), plot(t,dem), title('Demodulated'), grid on

subplot(3,2,5), plot(f,AM_FFT), xlim([fc-3*fm fc+3*fm]), title('Sidebands'), grid on

subplot(3,2,6), bar([Pc Psb Psb]), set(gca,'XTickLabel',{'Carrier','USB','LSB'}),

title('Power'), grid on

fprintf('μ=%.2f | USB=%d Hz | LSB=%d Hz | Pc=%.2f W | Psb=%.3f

W\n',mu,USB,LSB,Pc,Psb);

OUTPUT (GRAPH)

Calculations for Sideband Power and Frequencies in AM

Given:

· Carrier Frequency, Hz

· Modulating Frequency, Hz

· Carrier Power, W

· Modulation Index, (100% modulation)

Result:

The AM signal was successfully generated and analyzed in MATLAB.

Objective 2:

To determine and verify the upper sideband (USB) and lower sideband (LSB) frequencies of the

AM signal using frequency spectrum analysis in MATLAB.

Upper Sideband Frequency (USB)

Lower Sideband Frequency (LSB)

Result:

i) Upper Sideband Frequency (USB) = 55 Hz

ii) Lower Sideband Frequency (LSB) = 45 Hz

Objective 3:

To calculate the carrier power, sideband power, and total transmitted power of the AM signal for

a given modulation index using MATLAB.

Sideband Power

For an AM signal,

Substituting and W:

Therefore,

● Upper Sideband Power (USB Power) = 0.125 W

● Lower Sideband Power (LSB Power) = 0.125 W

4. Total Sideband Power

5. Total Transmitted Power

Result

i) Carrier Power = 0.5 W

ii) Upper Sideband Power = 0.125 W

iii) Lower Sideband Power = 0.125 W

iv) Total Sideband Power = 0.25 W

v) Total Transmitted Power = 0.75 W